Boolean Expression

A boolean expression is a logical statement that evaluates to either true or false. These expressions are fundamental in computer programming for decision-making and control flow, and they can be formed by comparing values with operators like >, <, ==, or by combining them with logical operators such as and, or, and not.

What they are

- True or false:

The result of a boolean expression is always one of two possible values: true or false.

- Logical statements:

They represent a logical statement or question that can be answered with a yes/no or true/false response.

- Used in programming:

They are used to control the flow of a program, allowing it to make decisions based on whether a condition is met.

How they work

- Relational operators:

You can create boolean expressions by comparing two values using relational operators:

  - > (greater than)

  - < (less than)

  - >= (greater than or equal to)

  - <= (less than or equal to)

  - == (equals)

  - != (not equals)

  - Example: 5 > 3 evaluates to true.

# Algebra Boolean Truth Tables

Boolean Algebra Expressions can be used to construct digital logic truth tables for their respective functions

As well as a standard Boolean Expression, the input and output information of any Logic Gate or circuit can be plotted into standard Boolean Algebra truth tables to give a visual representation of the switching function of the system.

The table used to represent the boolean expression of a logic gate function is commonly called a Truth Table. A logic gate truth table shows each possible input combination to the gate or circuit with the resultant output depending upon the combination of these input(s).

For example, consider a single 2-input logic circuit with input variables labeled as A and B. There are “four” possible input combinations or 22 of “OFF” and “ON” for the two inputs. However, when dealing with Boolean expressions and especially logic gate truth tables, we do not general use “ON” or “OFF” but instead give them bit values which represent a logic level “1” or a logic level “0” respectively.

Then the four possible combinations of A and B for a 2-input logic gate is given as:

- Input Combination 1. – “OFF” – “OFF” or ( 0, 0 )

- Input Combination 2. – “OFF” – “ON” or ( 0, 1 )

- Input Combination 3. – “ON” – “OFF” or ( 1, 0 )

- Input Combination 4. – “ON” – “ON” or ( 1, 1 )

Therefore, a 3-input logic circuit would have 8 possible input combinations or 23 and a 4-input logic circuit would have 16 or 24, and so on as the number of inputs increases. Then a logic circuit with “n” number of inputs would

have 2n possible input combinations of both “OFF” and “ON”.

So in order to keep things simple to understand, in this tutorial we will only deal with standard 2-input type logic gates, but the principals are still the same for gates with more than two inputs.

Then the Truth tables for a 2-input AND Gate, a 2-input OR Gate and a single input NOT Gate are given as:

Boolean Algebra Truth Tables For A 2-input AND Gate

For a 2-input AND gate, the output Q is true if BOTH input A “AND” input B are both true, giving the Boolean Expression of: ( Q = A and B ).

| Symbol | Truth Table |  |  |
| --- | --- | --- | --- |
|  | A | B | Q |
|  | 0 | 0 | 0 |
|  | 0 | 1 | 0 |
|  | 1 | 0 | 0 |
|  | 1 | 1 | 1 |
| Boolean Expression Q = A.B | Read as A AND B gives Q |  |  |

## Boolean Algebra Truth Tables For A 2-input OR Gate

For a 2-input OR (Inclusive-OR) gate, the output Q is true if EITHER

input A “OR” input B is true, giving the Boolean Expression of: ( Q = A or B ).

| Symbol | Truth Table |  |  |
| --- | --- | --- | --- |
|  | A | B | Q |
|  | 0 | 0 | 0 |
|  | 0 | 1 | 1 |
|  | 1 | 0 | 1 |
|  | 1 | 1 | 1 |
| Boolean Expression Q = A+B | Read as A OR B gives Q |  |  |

## Boolean Algebra Truth Tables For The NOT Gate

For a single input NOT (Inverter) gate, the output Q is ONLY true when the input is “NOT” true, the output is the inverse or complement of the input giving the Boolean Expression of: ( Q = NOT A ).

| Symbol | Truth Table |  |
| --- | --- | --- |
|  | A | Q |
|  | 0 | 1 |
|  | 1 | 0 |
| Boolean Expression Q = NOT A or A | Read as inversion of A gives Q |  |

## 2-input NAND (Not AND) Gate

For a 2-input NAND gate, the output Q is NOT true if BOTH input A and input B are true, giving the Boolean Expression of: ( Q = not(A AND B) ).

| Symbol | Truth Table |  |  |
| --- | --- | --- | --- |
|  | A | B | Q |
|  | 0 | 0 | 1 |
|  | 0 | 1 | 1 |
|  | 1 | 0 | 1 |
|  | 1 | 1 | 0 |
| Boolean Expression Q = A .B | Read as A AND B gives NOT-Q |  |  |

## 2-input NOR (Not OR) Gate

For a 2-input NOR gate, the output Q is true if BOTH input A and input B are NOT true, giving the Boolean Expression of: ( Q = not(A OR B) ).

| Symbol | Truth Table |  |  |
| --- | --- | --- | --- |
|  | A | B | Q |
|  | 0 | 0 | 1 |
|  | 0 | 1 | 0 |
|  | 1 | 0 | 0 |
|  | 1 | 1 | 0 |
| Boolean Expression Q = A+B | Read as A OR B gives NOT-Q |  |  |

As well as the standard logic gates there are also two special types of logic gate function called an Exclusive-OR Gate and an Exclusive-NOR Gate. The Boolean expression to indicate an Exclusive-OR or Exclusive-NOR function is to a symbol with a plus sign inside a circle, ( ⊕ ).

The switching actions of both of these types of gates can be created using the above standard logic gates. However, as they are widely used functions they are now available in standard IC form and have been included here as reference.

## 2-input EX-OR (Exclusive OR) Gate

For a 2-input Ex-OR gate, the output Q is true if EITHER input A or if

input B is true, but NOT both giving the Boolean Expression of: ( Q = (A and NOT B) or (NOT A and B) ).

| Symbol | Truth Table |  |  |
| --- | --- | --- | --- |
|  | A | B | Q |
|  | 0 | 0 | 0 |
|  | 0 | 1 | 1 |
|  | 1 | 0 | 1 |
|  | 1 | 1 | 0 |
| Boolean Expression Q = A ⊕ B |  |  |  |

## 2-input EX-NOR (Exclusive NOR) Gate

For a 2-input Ex-NOR gate, the output Q is true if BOTH input A and

input B are the same, either true or false, giving the Boolean Expression of: ( Q = (A and B) or (NOT A and NOT B) ).

| Symbol | Truth Table |  |  |
| --- | --- | --- | --- |
|  | A | B | Q |
|  | 0 | 0 | 1 |
|  | 0 | 1 | 0 |
|  | 1 | 0 | 0 |
|  | 1 | 1 | 1 |
| Boolean Expression Q = A ⊕ B |  |  |  |